L20 DOORS / SHUTTERS / HATCHES

610 ROLLER SHUTTERS

INSULATED ROLLER SHUTTER (SII85)

- MANUFACTURER

BL Shutters Ltd, Carn Rd, Carn Industrial Estate, Portadown, Co Armagh.

Telephone: 028 38333551. Fax: 028 38350204.

Email: sales@blshutters.co.uk . Website: www.blshutters.co.uk

- APPLICATION

WAREHOUSING, FACTORIES, FARM BUILDINGS

An insulated version of the commonly used Industrial Roller Shutter with a proven design over decades. The lath design incorporates an insualting thermal foam fill infill with an insulating value of 0.69 W/M.Sq.k and an acoustic value of 18 DBA to offer a smooth running Roller Shutter that conserves heat and reduces 
noise. Available in coloured polyester powder coating,plastisol or standard galvanised finish.

- SIZE

Maximum Size recommended 7000mm x 7000mm. (Larger on request)

- LEGISLATION COMPLIANT

Adhering to new legislation requiring all roller shutters to comply with the Construction Products Directive (C.P..D), all doors manufactured by BL Shutters are designed, manufactured and independently tested to meet the requirements of BS EN 13241 : 2003 & Construction Products Directive 89/106/EFC.

- FIXING

Vertical face or reveal fixed.

- BARREL

For openings up to 6 metres wide. Constructed from 139mm outside diameter x 126mm inside diameter steel tube mounted on 25mm steel shaft, with steel bearings at each end. Incorporating high speed bearings. Finish one coat of Red Oxide paint.

- CURTAIN

Constructed from 85mm twin skinned interlocking galvanised steel lathes. Ribbed for strength. 
Each lath infilled with CFC free insulation offering thermal values 0.69W/m2k and an acoustic value of 18DBA. Curtain finished with a galvanised bottom rail with integral rubber seal.

- SIDE GUIDES

Constructed from 85mm x 3mm galvanised steel guide complete with self locking. 
Brush strip carrier including brush seals.

- ELECTRICAL OPERATION / ELECTRICITY SUPPLY

By means of 1/2 HP up to 2 HP geared motor complete with rotary limits and emergency hand
mechanism. 3 push button controls as standard. Electricity supply 240 volt 1 phase and 415 volt 3 
phase.

Safety Brake fitted as standard.

- OPTIONAL EXTRAS

Plastisol Laths, Powder Coated Laths, Wicket Gate, Hood, Heavy Duty Weather Seal, Remote Control, Fascia, Vision Panels.